Effect of TIG Pulse spot welding process on AISI 1008 grade Low carbon steel lap Joints

1B. Vadivelu, 2S.Senthoor Selvan 3S.T.Selvamani, 4V. Balasubramanian

1Assistant Professor,3Professor,Department of Mechanical Engineering, Chennai Institute of Technology, Chennai, Tamil Nadu, India.

2UG Student, Dept. Of Manufacturing Engineering, Central Institute of Petrochemicals Engineering & Technology (CIPET), Chennai, Tamil Nadu, India.

4Professor, Centre for Metal Joining and Research, Department of Manufacturing Engineering, Annamalai University, Chidambaram, Tamil Nadu, India.

1E-mail: vadivelb@citchennai.net

Abstract:

The Fighting Drone development have enormous insight in defence sector worldwide. As well as, choosing materials for this unmanned aerial vehicle is the challenging process because it has to withstand in various aerodynamic condition. Some subassemblies are made up of low carbon steel due to its flexural stability which is highly required for fighting drones. Also, subassemblies are need sound joints to with stand the shear and impact force. TIG Pulse spot welding is an emerging welding process to join the thin sheets materials due to its high performance in dynamic circumstances. Therefore in this research work, AISI 1008 grade low carbon steel joints are fabricated by TIG Pulse spot welding to understand its mechanical and metallurgical properties. Also, the results are compared with conventional TIG spot welding process and it shows that, 18 % higher in tensile shear force load for TIG Pulse spot welded lap joints when compared to TIG spot welded joints. In addition, the shear fracture surface showed ductile mode with more cup and cone shapes in TIG Pulse spot joints.

Keywords: TIG Pulse spot welding, Low carbon steel, Unmanned Aerial Vehicle