Study on repair weldments of AA6082-T6 alloy and analysis on mechanical properties and metallurgical transformations

Deepak Adakmol1,a,  Ayush Verma1, Amit Mokashi1

1Larsen & Toubro Limited – Precision Engineering & Systems, Talegaon

aE-mail : deepak.adakmol@larsentoubro.com; Contact : 9819556615

### Abstract

This paper covers repair welding of AA6082-T6, a medium-strength Al–Mg–Si alloy, which is widely used in structural, transportation, marine and aerospace industries due to its excellent combination of strength, corrosion resistance, and formability. However, welding this precipitation-hardenable alloy presents significant metallurgical challenges. The high thermal gradients during welding cause dissolution and coarsening of the strengthening Mg₂Si precipitates, leading to softening in the Heat-Affected Zone (HAZ) and a consequent reduction in joint efficiency. Additionally, issues such as hot cracking, porosity, and distortion complicate the welding and repair processes. A comparison study was done between AA6082-T6 welded, post-weld artificial aging (carried out at 180°C for 4 hours) and same material after repair welding. In repair welding, repeated thermal cycles exacerbate microstructural degradation and residual stresses, demanding precise control of process parameters and heat treatment schedules. The study underlines the critical relationship between welding thermal history, precipitation behavior, and mechanical performance, emphasizing the need for optimized welding to ensure reliable repair and service performance of AA6082-T6 components in aerospace applications.

Keywords: AA6082, Gas Metal Arc Welding, Repair & reclamation, Artificial Aging, Metallurgical & mechanical properties, Mg₂Si Precipitates, Heat-Affected Zone.