FABRICATION OF FUNCTIONALLY GRADED ALUMINIUM ALLOY CYINDRICAL COMPONENTS USING COLD METAL TRANSFERRED WIRE ARC ADDITIVE MANUFACTURING TECHNIQUE

¹S.Nandha Madhavan*, ²E.Elambarithi, ³ R.Sudalaimani , 4S.Malarvizhi,

5V.Balasubramanian

¹,2 UG Studenr,3Research Scholar, 4,5Professor

1-5Centre for Materials Joining and Research (CEMAJOR),

Department of Manufacturing Engineering,

Annamalai University, Annamalai Nagar-608002

*Email: nandhamadhavans@gmail.com

Abstract: Additive manufacturing (AM) has emerged as a transformative technique for fabricating complex and near-net-shape metallic components with significant material efficiency and design flexibility. Among the various AM processes, Wire Arc Additive Manufacturing (WAAM) has gained increasing attention due to its high deposition rates, cost efficiency, and capability to produce large metallic structures. The Cold Metal Transfer (CMT) variant of WAAM, in particular, offers improved control over heat input and spatters formation, making it highly suitable for aluminium alloy processing where issues of porosity, solidification cracking, and distortion are prevalent. Aluminium alloys are extensively used in aerospace, automotive, and marine sectors owing to their high strength-to-weight ratio, corrosion resistance, and excellent thermal conductivity. The present work investigates the mechanical behaviour and micro structural characteristics of functionally graded aluminium alloy cylinders fabricated by Cold Metal Transfer–Wire Arc Additive Manufacturing (CMT-WAAM) using three different grades of filler wires. Cylinders were deposited layer by layer with different filler compositions, including AA1100, AA4043, and AA5356, to study the influence of compositional transitions on the final component integrity. Detailed metallographic characterization was carried out to reveal grain morphology, interlayer fusion, and the evolution of micro structural features at the alloy interfaces. Mechanical properties were assessed through tensile testing, hardness measurements, and fracture surface analyses to correlate the micro structural findings with performance. The results highlight the role of filler wire compatibility and solidification behaviour in governing porosity formation, interface bonding quality, and overall mechanical response. This study provides insights into optimizing multi-wire CMT-WAAM strategies for functional aluminium alloy components, paving the way for improved applications in lightweight structural systems.

Keywords: Wire arc additive manufacturing, Cold metal transfer, Aluminium alloys, Mechanical properties, Microstructure