In Situ Modification of Centrifugal Extractor Banks and Associated Piping in Site Constraint to Achieve Stringent Quality Welding

K. Palani, G Elaiyaraja, V John Hunter, N Subramanian, A.K. Karthi and M Dhananjeyakumar

Reprocessing Maintenance Division (Mechanical),

Reprocessing Group

Indira Gandhi Centre for Atomic Research (IGCAR), Kalpakkam-603102

Email: palani@igcar.gov.in &Mobile No.: 09443881219

Abstract

Centrifugal Extractor equipment is considered as the heart of the plant in the Fast Reactor Fuel Reprocessing plant for separation of Uranium, Plutonium and fission product by extraction methods using Purex Process in the spent fuels of irradiated fuels. The equipment is remotely operated and maintenance of centrifugal extractor bowls and electrical driven motors is facilitated using master slave manipulators and in-cell crane due to high radiation and contamination problems.

As per initial design, There were two rows of Centrifugal extractors having 1 to 10 and 11 to 20 stages which were mounted one behind the other with an elevation difference of 50 mm between the two rows, where the rear row would be at higher elevation. This arrangement is for better viewing and access to the pair of fasteners securing each bowl with the base as a part of maintenance. In the pilot plant of CORAL, the CE banks were erected in such a way that the aqueous and organic process stream from the last stage of first row would feed the first stage of second row of CE bank and this transfer of aqueous and organic would happen by gravity as per the process flow sheet. The remote maintenance of CORAL CE bowls and electrical motors was found difficult because of lack of visibility of 3 out of 4 rows of fasteners securing the CE motors, hence the difficulty in accessing the fasteners remotely. In the above backdrop the DFRP CE banks were modified with an elevation difference of 50mm between the two rows of each bank for easy accessibility of CE bowls and electrical motors from maintenance point of view. Because of the elevation difference between the first row &second row of CE banks, the feeding of process fluid from last stage of first row to the first stage of second row, which has to happen by gravity could not be achieved. Hence in order to overcome this issue, an intermediate transfer airlift was introduced in the demonstration plant.

During ACID TBP runs, it was observed that the introduction of intermediate transfer airlift resulted in chocking problems witnessed in the piping associated with intermediate transfer airlift. Due to the chocking problems, the intermediate air lift could not be operated, due to which the extraction cycle could not be completed. Hence, there was a need to eliminate the intermediate airlift transfer system & the required modification of all the Centrifugal Extractors wherein both the banks had to be restored at same elevation as that of the CE in the pilot plant, CORAL. The modification has to be carried out in-situ with all the CE banks in erected condition, which are connected with cluster of piping just 50 mm above the floor of the contactor cell containment boxes. Moreover, as per the CE layout, the organic and aqueous solution feed pipe running between the last stage of first row and bowl and the first stage of second row are oriented at a bare minimum distance of 80mm. The short length of the feed pipe, whose fabrication & erection exacerbated by multi plane bend to match with the outlet & inlet ports of the two stages discussed above were successfully welded in-situ in a short time in tandem with the quality requirements.

In situ modification of the above-mentioned works required appropriate sequencing of fabrication/Welding/NDE activities in order to meet the stringent quality requirements amid site constraints. The modification of the CE of DFRP was completed successfully within the scheduled time period without affecting the project commissioning schedule and the modified CE were successfully tested for performance using acid TBP runs and Uranium runs. This paper will narrate the experience during in-situ modification.

Key words: GTAW process, in-situ modifications, pipe welds, centrifugal extractor  and sequence welding.