Welding Qualification of dissimilar weld joints for chemical reactors used for Hydrogen production

C. B. Rajeev*, R. K. Mourya, Parmanand Kumar, Anant Kumar, Hemant Kumar, T. Saravanan, M. Krishnamoorthy P. Sreenivasa Rao, S. Chandramouli and S. Sridhar

Indira Gandhi Centre for Atomic Research, Kalpakkam, Tamil Nadu- 603127, India

*Corresponding Author E-mail ID: rajeevcb@igcar.gov.in

The thermochemical cycle is a promising approach for generating clean hydrogen. This technology requires a high-temperature heat source and generates hydrogen through water splitting via a series of chemical reactions involving copper and chlorine compounds, all of which are conducted in a closed loop. During the current development phase, the heat requirement is met by high-temperature, non-radioactive secondary sodium from a sodium-cooled fast reactor. Additionally, the superheated steam is used as a water source needed for hydrogen production. Nickel-based alloys, viz., alloy 625 (Inconel 625) and alloy C276 (Hastelloy C276) are selected as materials for constructing chemical reactors due to their excellent corrosion resistance at 500°C in molten CuCl and oxygen gas environment. Austenitic stainless steel grade 316L is utilised for the sodium system. However, the construction of chemical reactors and their subsequent integration with the secondary sodium system of the reactor requires dissimilar weld joints with stringent quality control. Combinations of dissimilar weld joints include welds of alloy 625 to alloy C276 and alloy 625 to SS 316L. To address this, welding procedures and performance qualifications have been established to fabricate weld joints that meet the specified acceptance criteria. This paper emphasises the welding procedures and performance qualification practices followed during the fabrication of thermochemical reactors and their integration with the sodium system.

Keywords: Chemical reactor, dissimilar weld joint, welding Qualification, Weld of alloy 625 with alloy C276 & SS 316L.