# Effect of Rolling Direction in Friction Stir Welding of AA6063 Plates

S M Rahul1, Harikrishnan R1, Jeffin Jose1, Sreeraj C R1,  Jithesh S R1*

* jithesh.me@adishankara.ac.in , 9846600403

1 Department of Mechanical Engineering, Adi Shankara Institute of Engineering and Technology

*Corresponding author

Abstract

Friction Stir Welding (FSW) is an innovative solid-state welding technique that was developed to address the limitations of traditional fusion welding methods. It is an environmental friendly process. Unlike fusion welding, FSW does not involve melting the materials but instead uses a rotating, non-consumable tool to generate friction and heat between the workpieces. The plasticized material is then mechanically stirred and create a welded joint. Its solid-state nature minimizes the formation of defects associated with traditional welding techniques, making it particularly suitable for aluminum alloys which are sensitive to heat. The ability of FSW to produce high-quality welds with minimal thermal distortion makes it an attractive option for joining AA6063 components in various applications, including construction, transportation, and other industries where lightweight and durable structures are essential. This research paper aims to provide optimize the mechanical properties of FSW joint of AA6063 plates. Experiments were conducted in two different orientation of the plates by varying input parameters such as tool rotational speed and welding speed. Microstructural anlysis was also conducted on the welded joint. The experimental results shows that, the welded joint perpendicular to the rolling direction of the plates with tool rotational speed 1400 rpm, welding speed 25 mm/min and axial force 8kN had the maximum tensile strength. The maxium value of tensile strength found to be 92.41 Mpa. While maximum hardness of 47.05 HV is obtained in the welded joint parallel to the rolling direction of the plates.

Keywords: FSW, tensile strength, hardness, tool rpm, welding speed, axial force.