Comparative Study of Finite Element Analysis of 316LN Stainless Steel Welding Using SYSWELD, FlexPDE and ANSYS

Mohammed Zubairuddin1, R. Ravikumar2, Shahid Tamboli3

1 Department of Mechanical Engineering, Aditya University, Surampalem.

zubersiddiqi09@gmail.com,

2Indira Gandhi Centre for Atomic Research, Kalpakkam-603102, Tamil Nadu, India

sravi@igcar.gov.in

3Department of Mechanical Engineering, Symbiosis Institute of Technology, Pune, India

shahidt@sitpune.edu.in

*Corresponding Author Email: zubersiddiqi09@gmail.com,

Abstract

In GTA welding, HAZ, FZ and Depth of penetration of the weld plate are essential features which depend upon the heat input parameters. Based on the heat input, the weld plate is subjected to thermal and mechanical deformation, causing further thermal cycles, residual stress and distortion in the weld plate. If not controlled, this stress can harm the mechanical performance of the welded components. Prediction of this thermal distribution at the design stage helps cut down the experimental and maintenance costs. FEM-based analysis is a favourite technique to predict thermal and stress distribution. The paper presented the FEM-based analysis of a 2 mm-thick 316LN steel plate welded using the GTA process. Three different Software ANSYS, SYSWELD and FlexPDE Software were used to analyze the thermal behaviour of GTA welding. The bead on the plate is carried out initially for heat source fitting for heat input parameters. The thermal cycle measured using a K-type thermocouple at a 10 mm distance is compared with three different software output values, and finally, a Mechanical analysis of the weld plate was carried out using SYSWELD software. Further predicted stress and distortion values are validated with measured stress values. Ultrasonic Testing Method is used for measuring the longitudinal stress value in the transverse direction of the plate. An electronic height gauge measures vertical displacement or distortion in the weld plate. Good agreement is observed between predicted and measured thermal cycles and stress values.