Study of metallurgical and mechanical properties of welded additively manufactured AlSi10Mg parts using GTAW and FSW

Vishvesh Badhekaa1, Ayush Reenaa2 and Krunal Mehtaa3

aDepartment of Mechanical Engineering, School of Technology,

Pandit Deendayal Energy University

1vishvesh.badheka@spt.pdpu.ac.in, 9824025256

2ayush.rmtmm22@sot.pdpu.ac.in, 9726643528

3krunal.mehta@sot.pdpu.ac.in, 9426337555

Abstract

Additive manufacturing (AM) is a crucial technique for the development of geometrically complex parts, with applications in the automotive, aerospace, space, and medical domains. One of the popular metal printing techniques is Laser Powder Bed Fusion (LPBF). LPBF has proven to be an effective technique for producing complex yet lightweight parts. The development of lightweight structures is best suited for aluminium alloys, with AlSi10Mg being a popular material used with LPBF because of its high fluidity and ease of manufacturing. AlSi10Mg has a low density and exhibits good static mechanical behaviour. When combined with the alloy’s advantage in design flexibility, the rapid solidification characteristic of the AM process yields a very fine microstructure and a noticeably high strength, making it a viable option for lightweight structural applications. The LPBF’s primary drawback is that it has a limited print size. Additionally, heat will be induced in the components as the sample’s height rises. This creates a need to join parts that are printed using LPBF, along with the need to join printed parts with geometrically simple wrought parts. Printed components can be welded together to create larger parts. The study aims to address this issue. In the study, printed parts are welded to one another using Gas Tungsten Arc Welding (GTAW) and Friction Stir Welding (FSW). The same techniques are also used to weld printed to wrought parts (Al6061) and wrought to wrought parts (Al6061) (for reference values). For such welded samples, the results for mechanical strength and microscopic structure are determined. The welded samples are also subjected to space environment post which their mechanical strengths are determined. The space environment included conditions for thermal-vacuum, thermal shock, humidity and radiation. The study thus contributes to establish the suitability of GTAW and FSW for welded AM components, particularly for space applications.

Key Words: Additive manufacturing, GTAW, FSW, Space applications