Mechanical and Metallurgical Characterization of Weld Joint Prepared with Hot Wire TIG using Inconel 617 Filler and SS 304L Base metal

Abhinav Garga1*, Karan Yadav2, Chandan Pandey2, Vishvesh Badheka2b

1,2Department of Mechanical Engineering, Indian Institute of Technology Jodhpur, Jodhpur 342030, India.

2bDepartment of Mechanical Engineering, SOT, PDEU, Gandhinagar, 382426, India.

*Corresponding author:  p23me0001@iitj.ac.in,

Contact detail: 7050186215

Abstract

This research article deals with the development of a weld joint using base metal (BM) SS 304L, with Inconel 617 filler. The automatic Hot Wire TIG machine with oscillation is employed to produce weld joint. The weld joint prepared with Hot Wire TIG with oscillation provides benefits such as dimensional accuracy and improved quality of weld joint. After completion of welding process dye-penetrant test (DPT) was utilized to identify the surface imperfection in weld joint and it noted that obtained weld joints are free from any imperfections. Further mechanical characterization has been performed on the weld joint which includes mechanical testing such as tensile test at room temperature and high temperatures (650°C and 700°C), Microhardness Test and Charpy impact toughness Test. It is identified that BM Inconel 617 (IN617) having higher hardness compared to SS 304L. The available phases are identified with the help of XRD and energy-dispersive X-ray spectroscopy (EDS) analysis.  Further optical microscopy (OM) and field emission scanning electron microscopy (FESEM) were utilized to capture high magnification image to identify the morphology of different zones. High magnification fractography image revels the mode of failure of tensile and Charpy impact test specimens at different test conditions. The results from these characterization techniques clearly show that the weld achieved by Hot Wire TIG with oscillation is superior in quality with better surface finish, dimensional accuracy with superior quality is feasible for high temperature applications.

Keywords: Hot Wire TIG, Inconel 617, SS 304L.