Optimizing Outer Hull Fabrication in Shipbuilding: A Strategy for Enhanced Welding Automation and Workflow Efficiency

Parthkumar Sapariyaa, Avinash Pandeyb, B Kiran Kumarc

a,b,c Larsen & Toubro Limited, Visakhapatnam, 530014, India.

Email: parthkumar.sapariya@larsentoubro.com,

avinash.pandey2@larsentoubro.com

kiran.kumar@larsentoubro.com

ABSTRACT

Modern Ship structures comprise of Complex & composite systems integrated with modules such as double bottoms, bulkheads, and decks, all contributing to overall stability and integrity. Central to this construction, the Outer Hull (OH) acts as the principal load-bearing envelope, securing water-tightness and providing an optimized hydrodynamic profile for propulsion. Fabrication of this OH Panel typically utilizes steel plates ranging from 8 to 26 mm thickness, strengthened by longitudinal stiffeners and frame brackets which are extensively welded to form large, rigid assemblies. This fabrication process poses significant difficulties due to extensive weld seams across complex geometrical profiles, restricted areas for welders, and a reliance on a pool of highly skilled workers.

The OH panel manufacturing sequence is comprehensive, involving plate cutting, rolling, sub-assembly followed by final carcass formation. Out of all the processes, data reveals that welding remains the singular critical bottleneck, directly dictating project schedules. In a typical OH panel section, sub-assemblies account for nearly 65% and assemblies 35% of fabrication workload. The dominant weld types are:

- T-Joints (Stiffener/Web to Plate): 45% - 55%

- Butt Joints (Plate-to-Plate joints): 30% - 40%

- Complex Profiled Joints (Curved joints, Corner joints): 10% - 15%

Stringent Quality requirements for these joints lead to frequent schedule overruns, highlighting the need for workflow innovation in welding.

This case-study paper demonstrates a planning for integrating advanced automation and track-based welding systems focused on high-impact joint categories: butt joints, T-stiffener welds, and complex sub-assemblies. This approach aims to achieve Process Stability and Weld Quality Consistency compared to conventional manual welding 	techniques.

The application of mechanized welding solutions and optimized workflow is projected to realize a notable 40% - 50% efficiency gain in the OH panel fabrication cycle. This study contributes a vital, quantified framework for achieving production acceleration, strategically addressing the challenge of welder scarcity, and guaranteeing the sustained operational reliability and high structural integrity essential for modern Shipbuilding programs.

Keywords: Shipbuilding, Outer Hull, DMR Steels, GMAW, Industry 5.0, Cobot, Robot, Flextrack, ArcRover, Productivity, Quality, Radiographic test.