Corrosion Behaviour of CMT-WAAM Fabricated Inconel 625 Hollow Cylinder

K S Sandesh, Godwin Mathew, Manu Harilal, Anoop K Unni *

Department of Metallurgical and Materials Engineering, Amal Jyothi College of Engineering, Kanjirappally, Kerala, 686518, India.

*Corresponding author: anoopkunni@amaljyothi.ac.in

Ph: +91 9447768819

Abstract

Inconel 625 is a nickel-based superalloy that is popular in marine, offshore and high-temperature applications due to its exceptional resistance to pitting, crevice attack and chloride-induced corrosion. Wire Arc Additive Manufacturing (WAAM) notably using the Cold Metal Transfer (CMT) process offers an efficient route to fabricate large hollow cylindrical components with minimal material waste. However, the layer-by-layer deposition inherent in WAAM can cause microstructural differences that impair corrosion performance. This study investigates how post-deposition heat treatment affects the microstructure, mechanical characteristics and corrosion response of Inconel 625 cylinders made with CMT-WAAM. Electrochemical studies, including open-circuit potential measurements, electrochemical impedance spectroscopy and potentiodynamic polarization tests, were carried out in a chloride-rich solution typical of marine environments. The results reveal that heat treatment minimizes dendritic segregation, enhances microstructural homogeneity, and stabilizes strengthening phases, all of which improve electrochemical stability. When compared to their as-fabricated counterparts, heat-treated samples showed lower corrosion current density, higher polarization resistance and impedance values, and a more robust passive film behaviour. Overall, the study demonstrates that appropriate thermal post-processing significantly improves the corrosion resistance and operational dependability of WAAM-built Inconel 625 components, making them suitable for use in challenging marine applications.

Keywords: Wire Arc Additive Manufacturing, Cold metal transfer, Inconel 625, corrosion resistance, heat treatment